Command Protocol Specification

AI Learner Rover Driver — Laptop App ↔ ESP32 Rover

1  Overview & Design Principles

This document specifies the messaging protocol between the laptop host application and the ESP32 rover. All messages are JSON objects exchanged over a single persistent WebSocket (TCP) connection.

- Persistent transport. A single WebSocket connection carries all traffic in both directions. Because TCP guarantees ordered, reliable delivery, the protocol does not add its own low-level delivery acknowledgements; application-level ACKs relate to task completion, not packet receipt.

- Compact JSON. Field names are kept short and messages minimal, because the ESP32-WROOM has limited RAM and flash. Verbose keys are avoided (e.g. “op” not “operation”).

- Every message has a type. A single field, “t”, identifies the message type so the receiver can dispatch it. All other fields depend on the type.

- Path-based driving. Driving is sent as a whole plan (a path), not one command at a time, terminated by STOP. The rover executes the path in order and reports progress.

2  Message Envelope

Every message is a JSON object with a short type field “t”. Optional “id” correlates a path and its ACKs.

{ "t": "<type>", ... type-specific fields ... }

3  Laptop → Rover Messages

| type | Purpose | Key fields |
| --- | --- | --- |
| path | A full driving plan: an ordered list of commands ending in STOP. | cmds: [ ... ] |
| heading | Reply to a rover heading request (from the overhead camera). | deg |
| telem | Telemetry on/off switch. Rover decides send rate while on. | on: true¦false |
| speed | Debug/tuning: set the base motor speed. | base: 0-255 |
| pid | Debug/tuning: set PID gains separately. | kp, ki, kd |

3.1  Driving path (the core message)

A path is an ordered list of commands the rover executes in sequence. It always ends with STOP, which signals the end of the path. Each command is a compact [op, val] pair.

{
  "t": "path",
  "id": 7,
  "cmds": [
    ["TURN", -30],      // 30 deg left
    ["FWD", 200],       // forward 200 mm
    ["STOP"]            // end of path
  ]
}

Supported commands:

| op | val | Meaning |
| --- | --- | --- |
| FWD | millimetres | Drive straight forward val mm. |
| BACK | millimetres | Drive straight backward val mm. |
| TURN | degrees (+right / -left) | In-place turn by val degrees. |
| STOP | (none) | End of path. Halt and hold. |

3.2  Heading reply

Sent in response to a rover heading request. The heading originates from the overhead camera on the laptop.

{ "t": "heading", "deg": 92.5 }

3.3  Telemetry switch

Telemetry works as an on/off switch. The laptop turns it on; the rover then decides when/how often to send samples. The laptop turns it off to stop the stream.

{ "t": "telem", "on": true }
{ "t": "telem", "on": false }

3.4  Debug / tuning (base speed & PID)

Two settings messages support debugging and tuning. Base speed and PID gains are set separately.

{ "t": "speed", "base": 180 }
{ "t": "pid", "kp": 1.2, "ki": 0.0, "kd": 0.4 }

4  Rover → Laptop Messages

| type | Purpose | Key fields |
| --- | --- | --- |
| ack | One command completed, OR the whole path completed. | of, id |
| nack | A command could not complete (e.g. obstacle detected). | id, reason |
| telem | Telemetry sample (only while telemetry is on). | vbat, soc, dist |
| req_heading | Rover asks the laptop for its current heading. | (none) |

4.1  Acknowledgements (ACK)

The rover sends an ACK when it believes it has finished each individual command, and a final ACK when the whole path is done. The “of” field distinguishes the two; “id” echoes the path id.

{ "t": "ack", "of": "cmd",  "id": 7, "i": 1 }   // finished command index 1
{ "t": "ack", "of": "path", "id": 7 }            // finished whole path

4.2  Negative acknowledgement (NACK)

If a command cannot be completed — for example, the safety layer detects an obstacle — the rover sends a NACK naming the command index and the reason. The path is halted.

{ "t": "nack", "id": 7, "i": 1, "reason": "obstacle" }

4.3  Telemetry sample

While telemetry is on, the rover streams samples at a rate of its own choosing. Contents: battery voltage, estimated state of charge, and the ultrasonic distance measurement.

{ "t": "telem", "vbat": 7.82, "soc": 74, "dist": 14 }

4.4  Heading request

The rover can request its current heading from the laptop (sourced from the overhead camera). The laptop replies with a §3.2 heading message.

{ "t": "req_heading" }

5  Message Flow (typical navigation cycle)

- 1. Laptop assembles a world-state, the LLM plans, and the laptop sends one “path” message (a full plan ending in STOP).

- 2. Rover executes each command in order. After each, it sends an ACK (of:“cmd”). It may send req_heading mid-path and receive a heading reply.

- 3. If the safety layer trips (obstacle), the rover sends a NACK and halts; the laptop re-plans.

- 4. On finishing the last command before STOP, the rover sends a final ACK (of:“path”).

- 5. At any time the laptop may toggle telemetry on/off; while on, the rover streams telemetry samples the laptop uses to update its displays.

6  Design Notes & Rationale

- Why a whole path, not one command at a time: the LLM plans a sequence; sending it as one package lets the rover execute open-loop between world-state snapshots (plan-then-execute), reducing round-trips.

- Why STOP terminates a path: it gives the interpreter an unambiguous end-of-plan marker and a defined safe end state.

- Why the rover controls telemetry timing: the laptop only says on/off; the rover chooses the rate, avoiding a stream of laptop poll requests and letting the rover throttle to protect its loop.

- Why no low-level ACKs: the WebSocket/TCP layer already guarantees reliable, ordered delivery, so application ACKs are reserved for task completion, not receipt.

- Robust interpreter: the ESP32 parser must reject malformed or unknown messages safely (ignore + optionally NACK) rather than wedging — important because the plan ultimately originates from an LLM.